LOVELY PROFESSIONAL UNIVERSITY

ACADEMIC TASK-2

CSE 307

Internet Working Essential

Submitted by:

Name: Sethukumar Moorthy

Registration number: 12307650

Roll Number: RK23FSB26

Section: K23FS

Submitted to: Bhupinder Kaur

GitHub-https://github.com/SETHUKUMAR1709/Netwok-Planning

### 1. Introduction

This report presents the physical and logical design of an office network infrastructure spread across 8 floors, with specific topologies and devices assigned to each floor. The network design emphasizes efficient communication, IP management, and the inclusion of essential network services.

### 2. Network Topology and Device Setup

Each floor contains 7 computers and uses a different topology as specified below:

| Floor | Topology Used | Devices Used |
| --- | --- | --- |
| 1–3 | Hybrid | Switch, Hub, PCs |
| 4–6 | Mesh | Switches, PCs |
| 7–8 | Bus | Hub, PCs |

#### 2.1 Devices Used

- Switch PT: Used in Hybrid and Mesh topologies to connect multiple systems.

- Hub PT: Used in Hybrid and Bus topologies for shared communication.

- Routers: Used to connect networks on different floors and implement routing.

- PCs & Laptops: End devices across all floors.

- Cables: Cross cables and straight-through cables for appropriate connections.

### 3. IP Addressing

We used FLSM for IP allocation, starting with the base network 192.122.5.0/24. Each floor is assigned a /28 subnet (16 IPs), which is enough for 7 computers and networking devices. The new Subnet Mask is 255.255.255.240 (/28).

| Subnet No. | Network Address | Usable IP Range | Broadcast Address |  |
| --- | --- | --- | --- | --- |
| 1 | 192.122.5.0 | 192.122.5.1 – 192.122.5.14 | 192.122.5.15 |  |
| 2 | 192.122.5.16 | 192.122.5.17 – 192.122.5.30 | 192.122.5.31 |  |
| 3 | 192.122.5.32 | 192.122.5.33 – 192.122.5.46 | 192.122.5.47 |  |
| 4 | 192.122.5.48 | 192.122.5.49 – 192.122.5.62 | 192.122.5.63 |  |
| 5 | 192.122.5.64 | 192.122.5.65 – 192.122.5.78 | 192.122.5.79 |  |
| 6 | 192.122.5.80 | 192.122.5.81 – 192.122.5.94 | 192.122.5.95 |  |
| 7 | 192.122.5.96 | 192.122.5.97 – 192.122.5.110 | 192.122.5.111 |  |
| 8 | 192.122.5.112 | 192.122.5.113 – 192.122.5.126 | 192.122.5.127 |  |
| 9 | 192.122.5.128 | 192.122.5.129 – 192.122.5.142 | 192.122.5.143 |  |
| 10 | 192.122.5.144 | 192.122.5.145 – 192.122.5.158 | 192.122.5.159 |  |
| 11 | 192.122.5.160 | 192.122.5.161 – 192.122.5.174 | 192.122.5.175 |  |
| 12 | 192.122.5.176 | 192.122.5.177 – 192.122.5.190 | 192.122.5.191 |  |
| 13 | 192.122.5.192 | 192.122.5.193 – 192.122.5.206 | 192.122.5.207 |  |
| 14 | 192.122.5.208 | 192.122.5.209 – 192.122.5.222 | 192.122.5.223 |  |
| 15 | 192.122.5.224 | 192.122.5.225 – 192.122.5.238 | 192.122.5.239 |  |
| 16 | 192.122.5.240 | 192.122.5.241 – 192.122.5.254 | 192.122.5.255 |  |

### 4. Routing Setup

The routing between the floors is implemented using Dynamic Routing (RIP).

Routing Configuration Steps:

- Routers are configured using both CLI and Packet Tracer UI.

- RIP allows automatic route updates between floors.

- CLI Commands:

arduino

CopyEdit

Router> enable

Router# config terminal

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.122.5.0

### 5. Inter-Floor Communication

- Floors 1–6 are connected in a series using routers.

- Floors 6–8 are connected via an additional router.

- Communication was tested using the command:

php-template

CopyEdit

ping <target IP>

This confirmed successful inter-floor data transmission.

### 6. Server Configuration

Each floor has been assigned a specific network service hosted on a server. These servers are configured with static IPs from their floor subnet.

| Floor | Server Type | Purpose |
| --- | --- | --- |
| 4 | DHCP Server | Automatically assigns IP addresses to client devices (like PCs) on the network without manual configuration. |
| 5 | DNS Server | Translates domain names (like www.example.com) into IP addresses. This is essential for web browsing and email routing. |
| 6 | HTTP Server | Hosts web pages and delivers HTML content over the web using HTTP/HTTPS. Can simulate a company website or internal portal. |
| 7 | Mail Server | Handles sending and receiving of emails within the network. Works using SMTP for sending and POP3/IMAP for receiving. |

### 7. Full Network Topology

### 8. Floor-wise Layouts and Configurations

- Floors 1–3: Hybrid Topology

- Floors 4–6: Mesh Topology

- Floors 7–8: Bus Topology

- Router & PC Configuration:

- Working: